# Switch Abstraction Interface

# Change Proposal

| Title | MPLS |
| --- | --- |
| Authors | Mellanox |
| Status | In Review |
| Type | Standards Track |
| Created | 05/03/2017 |
| SAI-Version | 1.1.0 |

Contents

List of Changes	i

1	Overview	1

1.1	InSegment	1

1.2	OutSegment(SAI next hop)	1

1.3	LabelStack	2

1.4	Configuration examples	2

1.4.1	LSR flow	2

1.4.2	Ingress LER	2

1.4.3	Egress LER	2

2	Pipeline model	2

2.1	LSR flow	2

2.2	Ingress LER flow	2

2.3	Egress LER flow	2

2.4	Egress LER PHP flow	3

2.5	L3VPN	3

2.6	Segment routing	3

3	Specification	3

3.1	SAI object	3

3.2	SAI router interface	4

3.3	SAI next hop	7

3.4	SAI InSegment	8

3.5	MPLS API summary	11

4	Examples	11

# List of Changes

| Version | Changes | Name | Date |
| --- | --- | --- | --- |
| 0.9.2 | Base version |  | 2/11/2015 |
| 1.1.0 | Update |  | 05/03/2017 |

License

© 2014 Microsoft Corporation, Dell Inc., Facebook, Inc, Broadcom Corporation, Intel Corporation, Mellanox Technologies Ltd.

As of September 9, 2014, the following persons or entities have made this Specification available under the Open Web Foundation Final Specification Agreement (OWFa 1.0), which is available at http://www.openwebfoundation.org/legal/the-owf-1-0-agreements/owfa-1-0

Microsoft Corporation, Dell Inc., Facebook, Inc, Intel Corporation, Mellanox Technologies Ltd.

You can review the signed copies of the Open Web Foundation Agreement Version 1.0 for this Specification at http://opencompute.org/licensing/, which may also include additional parties to those listed above.

Your use of this Specification may be subject to other third party rights. THIS SPECIFICATION IS PROVIDED "AS IS." The contributors expressly disclaim any warranties (express, implied, or otherwise), including implied warranties of merchantability, noninfringement, fitness for a particular purpose, or title, related to the Specification. The entire risk as to implementing or otherwise using the Specification is assumed by the Specification implementer and user. IN NO EVENT WILL ANY PARTY BE LIABLE TO ANY OTHER PARTY FOR LOST PROFITS OR ANY FORM OF INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES OF ANY CHARACTER FROM ANY CAUSES OF ACTION OF ANY KIND WITH RESPECT TO THIS SPECIFICATION OR ITS GOVERNING AGREEMENT, WHETHER BASED ON BREACH OF CONTRACT, TORT (INCLUDING NEGLIGENCE), OR OTHERWISE, AND WHETHER OR NOT THE OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

THE FOLLOWING IS A LIST OF MERELY REFERENCED TECHNOLOGY: Microprocessor technology, semiconductor manufacturing technology, operating system technology (including without limitation networking operating system technology), emulation technology, graphics technology, video technology, integrated circuit packaging technology and the like, compiler technologies, object oriented technology, optical/RF communications technology including chip I/O and driver technology, bus technology, memory chip technology (including, without limitation, NAND memory, NOR memory, resistive RAM (RRAM), seek scan probe (SSP) memory, nonvolatile memory (including without limitation, memory based on chalcogenide materials, phase change memory (PCM), one or more stacked layers of memory cells, embedded PCM memories, non-volatile cache memory, solid state drives, SRAM, embedded DRAM, ferro-electric memory, and polymer memory)) and/or health-related and medical technology. IMPLEMENTATION OF THESE TECHNOLOGIES MAY BE SUBJECT TO THEIR OWN LEGAL TERMS.

# Overview

MPLS library provides an access to MPLS functionality of the switch. SAI provides tools to configure the switch to forward MPLS packets, create basic MPLS objects in HW, bind them together in order to create MPLS flow in HW. Also SAI provides ability to protect MPLS LSPs, terminate MPLS LSP and forward packets to IP router and encapsulate IP packets with MPLS.

Scope : IP MPLS MLPs tunnel will be added at latter stage

Three basic objects are defined in order to manage MPLS functionality:

-	InSegment

-	OutSegment

-	LabelStack

## InSegment

An InSegment defines incoming MPLS segment to a Label Switch Router (LSR), needed parameters and actions. This object represents an incoming label as it was created by signaling protocol, network management or node administrator and POP operation of the LSR.

The InSegment contains following parameters:

- Incoming label

- Number of pops

- Action

- Counter

InSegment can be bound to LabelStack or OutSegment in order to create a LSR flow in HW.

InSegment can be bound to multiple objects to create ECMP.

InSegment can be a termination point and cause packet re-lookup.

## OutSegment(SAI next hop)

An OutSegment defines outgoing parameters from Label Switch Router to a MPLS network. This object represents outgoing top MPLS label. This object performs PUSH and PHP operations.

OutSegment has following parameters:

-	labelStack object

-	Next IP hop

OutSegment can be bound to InSegment LSR flow  , and can be bound to route in in order to create ingress LER .

Several InSegments or LabelSteacks can be bound to same OutSegment to create goto-tunnel flow.

## LabelStack

A LabelStack defines a list of labels that are pushed to the packet beneath top label defined by OutSegment.

The LabelStack is a group of labels that can be bound to another objects in order to create LSR path or backup path.

## Configuration examples

### LSR flow

Label stack

In segment

Out segment

Router

Ingress LER

Label stack

Out segment

In segment

Egress LER

Out segment

# Pipeline model

## LSR flow

## Ingress LER flow

## Egress LER flow

## Egress LER PHP flow

## L3VPN

## Segment routing

# Specification

[This section describes the details of the proposed interface/API]

## SAI object

/**

* @brief Structure for bulk retrieval of objectids, attribute and values for

* each object-type. Key will be used in case of object-types not having

* object-ids.

*/

typedef struct _sai_object_key_t

{

union

{

sai_object_id_t           object_id;

sai_fdb_entry_t           fdb_entry;

sai_neighbor_entry_t      neighbor_entry;

sai_route_entry_t         route_entry;

sai_mcast_fdb_entry_t     mcast_fdb_entry;

sai_l2mc_entry_t          l2mc_entry;

sai_ipmc_entry_t          ipmc_entry;

sai_in_segment_entry      inseg_entry;

} key;

} sai_object_key_t;

## SAI router interface

typedef enum _sai_router_interface_type_t

{

/** Port or Lag Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_PORT,

/** VLAN Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_VLAN,

/** Loopback Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_LOOPBACK,

/** Loopback Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_MPLS_ROUTER

/** Sub port Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_SUB_PORT,

/** .1D Bridge Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_BRIDGE

} sai_router_interface_type_t;

/**

* @brief Routing interface attribute IDs

*/

typedef enum _sai_router_interface_attr_t

{

/**

* @brief Start of attributes

*/

SAI_ROUTER_INTERFACE_ATTR_START,

/* READ-ONLY */

/**

* @brief Virtual router id

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_VIRTUAL_ROUTER

*/

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID = SAI_ROUTER_INTERFACE_ATTR_START,

/**

* @brief Type

*

* @type sai_router_interface_type_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

*/

SAI_ROUTER_INTERFACE_ATTR_TYPE,

/**

* @brief Assosiated Port or Lag object id

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_PORT, SAI_OBJECT_TYPE_LAG

* @condition SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_PORT or SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_SUB_PORT

*/

SAI_ROUTER_INTERFACE_ATTR_PORT_ID,

/**

* @brief Assosiated Vlan

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_VLAN

* @condition SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_VLAN or SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_SUB_PORT

*/

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID,

/* READ-WRITE */

/**

* @brief MAC Address

*

* Not valid when #SAI_ROUTER_INTERFACE_ATTR_TYPE == #SAI_ROUTER_INTERFACE_TYPE_LOOPBACK)

* Default to #SAI_VIRTUAL_ROUTER_ATTR_SRC_MAC_ADDRESS if not set on create)

*

* @type sai_mac_t

* @flags CREATE_AND_SET

* @default attrvalue SAI_VIRTUAL_ROUTER_ATTR_SRC_MAC_ADDRESS

*/

SAI_ROUTER_INTERFACE_ATTR_SRC_MAC_ADDRESS,

/**

* @brief Admin V4 state

*

* @type bool

* @flags CREATE_AND_SET

* @default true

*/

/**

* @brief Assosiated Port or Lag object id

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_RIF,

* @condition SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_MPLS_ROUTER         */

SAI_ROUTER_INTERFACE_ATTR_RIF_ID,

SAI_ROUTER_INTERFACE_ATTR_ADMIN_V4_STATE,

/**

* @brief Admin V6 state

*

* @type bool

* @flags CREATE_AND_SET

* @default true

*/

SAI_ROUTER_INTERFACE_ATTR_ADMIN_V6_STATE,

/**

* @brief MTU

*

* @type sai_uint32_t

* @flags CREATE_AND_SET

* @default 1514

*/

SAI_ROUTER_INTERFACE_ATTR_MTU,

/**

* @brief RIF bind point for ingress ACL object

*

* Bind (or unbind) an ingress acl table or acl group on a RIF. Enable/Update

* ingress ACL table or ACL group filtering by assigning a valid object id.

* Disable ingress filtering by assigning SAI_NULL_OBJECT_ID in the

* attribute value.

*

* @type sai_object_id_t

* @flags CREATE_AND_SET

* @objects SAI_OBJECT_TYPE_ACL_TABLE, SAI_OBJECT_TYPE_ACL_TABLE_GROUP

* @allownull true

* @default SAI_NULL_OBJECT_ID

*/

SAI_ROUTER_INTERFACE_ATTR_INGRESS_ACL,

/**

* @brief RIF bind point for egress ACL object

*

* Bind (or unbind) an egress acl table or acl group on a RIF. Enable/Update

* egress ACL table or ACL group filtering by assigning a valid object id.

* Disable egress filtering by assigning SAI_NULL_OBJECT_ID

* in the attribute value.

*

* @type sai_object_id_t

* @flags CREATE_AND_SET

* @objects SAI_OBJECT_TYPE_ACL_TABLE, SAI_OBJECT_TYPE_ACL_TABLE_GROUP

* @allownull true

* @default SAI_NULL_OBJECT_ID

*/

SAI_ROUTER_INTERFACE_ATTR_EGRESS_ACL,

/**

* @brief Packet action when neighbor table lookup miss for this router interface

*

* @type sai_packet_action_t

* @flags CREATE_AND_SET

* @default SAI_PACKET_ACTION_TRAP

*/

SAI_ROUTER_INTERFACE_ATTR_NEIGHBOR_MISS_PACKET_ACTION,

/**

* @brief V4 Mcast enable

*

* @type bool

* @flags CREATE_AND_SET

* @default false

*/

SAI_ROUTER_INTERFACE_ATTR_V4_MCAST_ENABLE,

/**

* @brief V6 Mcast enable

*

* @type bool

* @flags CREATE_AND_SET

* @default false

*/

SAI_ROUTER_INTERFACE_ATTR_V6_MCAST_ENABLE,

/**

* @brief End of attributes

*/

SAI_ROUTER_INTERFACE_ATTR_END,

/** Custom range base value */

SAI_ROUTER_INTERFACE_ATTR_CUSTOM_RANGE_START = 0x10000000,

/** End of custom range base */

SAI_ROUTER_INTERFACE_ATTR_CUSTOM_RANGE_END

} sai_router_interface_attr_t;

## SAI next hop

typedef enum _sai_next_hop_type_t

{

/** IP next hop */

SAI_NEXT_HOP_TYPE_IP,

/** MPLS(outsegment) next hop */

SAI_NEXT_HOP_TYPE_MPLS,

/** Tunnel next hop */

SAI_NEXT_HOP_TYPE_TUNNEL_ENCAP

} sai_next_hop_type_t;

/**

* @brief Attribute id for next hop

*/

typedef enum _sai_next_hop_attr_t

{

/**

* @brief Start of attributes

*/

SAI_NEXT_HOP_ATTR_START,

/**

* @brief Next hop entry type

*

* @type sai_next_hop_type_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

*/

SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_ATTR_START,

/**

* @brief Next hop entry ipv4 address

*

* @type sai_ip_address_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @condition SAI_NEXT_HOP_ATTR_TYPE == SAI_NEXT_HOP_TYPE_IP

*/

SAI_NEXT_HOP_ATTR_IP,

/**

* @brief Next hop entry router interface id

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_ROUTER_INTERFACE

*/

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID,

/**

* @brief Next hop entry tunnel-id

*

* @type sai_object_id_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @objects SAI_OBJECT_TYPE_TUNNEL

* @condition SAI_NEXT_HOP_ATTR_TYPE == SAI_NEXT_HOP_TYPE_TUNNEL_ENCAP

*/

SAI_NEXT_HOP_ATTR_TUNNEL_ID,

/**

* @brief  push label

*

* @type sai_object_list_t

* @flags MANDATORY_ON_CREATE | CREATE_ONLY

* @condition SAI_NEXT_HOP_ATTR_TYPE == SAI_NEXT_HOP_TYPE_MPLS

*/

SAI_NEXT_HOP_ATTR_LABELSTACK,

/**

* @brief End of attributes

*/

SAI_NEXT_HOP_ATTR_END,

/** Custom range base value */

SAI_NEXT_HOP_ATTR_CUSTOM_RANGE_START = 0x10000000,

/** End of custom range base */

SAI_NEXT_HOP_ATTR_CUSTOM_RANGE_END

} sai_next_hop_attr_t;

## SAI InSegment

/*

*  in_segment entry

*/

typedef struct _sai_in_segment_attribute_t

{

SAI_INSEG_ENTRY_ATTR_STAR;

/** number of pop   */

SAI_INSEG_ENTRY_NUM_OF_PO,

/**

* @brief Packet action

* @type sai_packet_action_t

* @flags CREATE_AND_SET

* @default SAI_PACKET_ACTION_FORWARD

*/

SAI_ INSEG _ENTRY_ATTR_PACKET_ACTION,

/**

* @brief Packet priority for trap/log actions

* @type sai_uint8_t

* @flags CREATE_AND_SET

* @default 0

*/

SAI_INSEG_ENTRY_ATTR_TRAP_PRIORITY,

/**

* @brief The next hop id

* @type sai_object_id_t

* @flags CREATE_AND_SET

* @default SAI_NULL_OBJECT_ID

*/

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID,

SAI_INSEG_ENTRY_ATTR_CUSTOM_RANGE_START,

SAI_ROUTE_ENTRY_ATTR_CUSTOM_RANGE_END

} sai_in_segment_attribute_t;

typedef struct _sai_in_segment_entry_t

{

sai_object_id_t switch_id;

sai_label_id_t  label;

}sai_in_segment_entry_t

/*

* Routine Description:

*    Create in_segment entry

*

* Arguments:

*    [in] in_segment_entry - InSegment entry

*    [in] attr_count - number of attributes

*    [in] attrs - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*

*

*/

typedef sai_status_t (*sai_create_in_segment_entry_fn)(

_In_ const sai_in_segment_entry_t* in_segment_entry,

_In_ uint32_t attr_count,

_In_ const sai_in_segment_attribute_t *attr_list

);

/*

* Routine Description:

*    Remove in_segment entry

*

* Arguments:

*    [in]  in_segment_entry - InSegment entry

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*

*/

typedef sai_status_t (*sai_remove_in_segment_entry_fn)(

_In_ const sai_in_segment_entry_t* in_segment_entry

);

/*

* Routine Description:

*    Set in_segment entry attribute value

*

* Arguments:

*    [in] in_segment_entry - InSegment entry

*    [in] attr - attribute

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_set_in_segment_entry_attribute_fn)(

_In_ const sai_in_segment_entry_t* in_segment_entry,

_In_ const sai_attribute_t *attr

);

/*

* Routine Description:

*    Get in_segment entry attribute value

*

* Arguments:

*    [in] in_segment_entry - InSegment entry

*    [in] attr_count - number of attributes

*    [inout] attrs - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_get_in_segment_entry_attribute_fn)(

_In_ const sai_in_segment_entry_t* in_segment_entry,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

typedef sai_status_t (*sai_get_out_segment_entry_attribute_fn)(

_In_ const sai_out_segment_id_t* out_segment_entry,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

## MPLS API summary

/*

* MPLS method table retrieved with sai_api_query()

*/

typedef struct _sai_mpls_api_t

{

sai_create_in_segment_entry_fn                      create_in_segment_entry;

sai_remove_in_segment_entry_fn                      remove_in_segment_entry;

sai_set_in_segment_entry_attribute_fn               set_in_segment_entry_attribute;

sai_get_in_segment_entry_attribute_fn               get_in_segment_entry_attribute;

} sai_mpls_api_t;

# Examples

## SW A configuration

//create VRF default

Default_vrf

*****

//Create rif_2

Rif_2 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE= SAI_ROUTER_INTERFACE_TYPE_VLAN

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=vlan 1

}

//Create rif_1

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE= SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 2

}

//Create rif_mpls

rif_mpls attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE= SAI_ROUTER_INTERFACE_TYPE_MPLS_ROUTER

}

//create neighbor entries

Host_A_neigh;

Host_A_neigh

{

ip_address=1.1.1.2

rif_id=rif_2

}

Host_A_neigh neighbor  attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=host_A

}

SW_B_neigh

{

ip_address=2.2.2.2

rif_id=rif_1

}

SW_B neighbor attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_B

}

//router

//ip2ME routes

route_entry

{

vr_id= default_vrf

destination=1.1.1.1/32

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SAI_SWITCH_ATTR_CPU_PORT

}

route_entry

{

vr_id= default_vrf

destination=2.2.2.1/32

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SAI_SWITCH_ATTR_CPU_PORT

}

//local route

route_entry

{

vr_id= default_vrf

destination=2.2.2.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=rif_1

}

route_entry

{

vr_id= default_vrf

destination=1.1.1.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=rif_2

}

//remote routes (ingress LER)

//route to host B network

//MPLS next hop

SW_B2_1 next hop  //net hop from SW A via SW B to 7.7.7.0/24

SW_B2C_next_hop attributes

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=2.2.2.2

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_1

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 3}

}

SW_B2_2 next hop//net hop from SW A via SW B to 6.6.6.0/24

SW_B2C_next_hop attributesv

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=2.2.2.2

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_1

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 2}

}

route_entry

{

vr_id= default_vrf

destination=6.6.6.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SW_B_2_next_hop

}

//route to host C network

route_entry

{

vr_id= default_vrf

destination=7.7.7.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SW_B_1_next_hop

}

//MPLS in segment enrty (egress LER)

In_seg {.label=label_1}

In_seg attributes

{

SAI_INSEG_ENTRY_NUM_OF_PO=1

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID=rif_mpls

}

## SW B configuration

//create VRF default

Default_vrf

*****

//Create rif_1

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE=SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 1

}

//Create rif_2

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE=SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 2

}

//Create rif_3

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE=SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 3

}

SW_D_neigh;

SW_A_neigh;

SW_C_neigh;

SW_A_neigh

{

ip_address=2.2.2.1

rif_id=rif_1

}

SW_A_neigh neighbor  attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_A

}

SW_C_neigh

{

ip_address=4.4.4.1

rif_id=rif_3

}

SW_C neighbor attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_C

}

SW_D_neigh

{

ip_address=3.3.3.1

rif_id=rif_2

}

SW_D neighbor attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_D

}

SW_A_1_next_hop attributes //net hop from SW B via SW A to 1.1.1.0/24

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=2.2.2.1

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_1

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 1}

}

SW_D_1_next_hop attributes //net hop from SW B via SW D to 7.7.7.0/24

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=3.3.3.1

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_2

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 1}

}

SW_C_1_next_hop attributes //net hop from SW B via SW C to 6.6.6.0/24

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=4.4.4.1

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_3

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 1}

}

//MPLS in segment enrty (egress LER)

In_seg {.label=label_3}

In_seg attributes

{

SAI_INSEG_ENTRY_NUM_OF_PO=1

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID=SW_D_1_next_hop

}

In_seg {.label=label_2}

In_seg attributes

{

SAI_INSEG_ENTRY_NUM_OF_PO=1

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID=SW_C_1_next_hop

}

In_seg {.label=label_1}

In_seg attributes

{

SAI_INSEG_ENTRY_NUM_OF_PO=1

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID=SW_A_1_next_hop

}

## SW C configuration

//create VRF default

Default_vrf

*****

//Create rif_1

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE=SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 1

}

//Create rif_2

Rif_1 attribute

{

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID=Default_vrf

SAI_ROUTER_INTERFACE_ATTR_TYPE=SAI_ROUTER_INTERFACE_TYPE_PORT

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID=port 2

}

SW_B_neigh;

SW_E_neigh;

SW_B_neigh

{

ip_address=4.4.4.2

rif_id=rif_2

}

SW_B_neigh neighbor  attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_B

}

SW_E_neigh

{

ip_address=5.5.5.2

rif_id=rif_1

}

SW_E neighbor attributes

{

SAI_NEIGHBOR_ENTRY_ATTR_DST_MAC_ADDRESS=SW_E

}

SW_B_1_next_hop attributes //net hop from SW C via SW B to 7.7.7.0/24

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=2.2.2.1

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_2

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 2}

}

SW_B_2_next_hop attributes //net hop from SW C via SW B to 1.1.1.0/24

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_MPLS

SAI_NEXT_HOP_ATTR_IP=3.3.3.1

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_2

SAI_NEXT_HOP_ATTR_LABELSTACK={.count=1 , .list[0]=label 1}

}

SW_E_1_next_hop attributes //net hop from SW C via SW E to 7.7.7.0/24  PHP

{

SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TYPE_IP

SAI_NEXT_HOP_ATTR_IP=5.5.5.2

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID=rif_1

}

//MPLS in segment enrty (egress LER)

In_seg {.label=label_1}

In_seg attributes

{

SAI_INSEG_ENTRY_NUM_OF_PO=1

SAI_INSEG_ENTRY_ATTR_NEXT_HOP_ID=SW_E_1_next_hop

}

//route

route_entry

{

vr_id= default_vrf

destination=1.1.1.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SW_B_2_next_hop

}

route_entry

{

vr_id= default_vrf

destination=7.7.7.0/24

}

route attributes

{

SAI_ROUTE_ENTRY_ATTR_NEXT_HOP_ID=SW_B_1_next_hop

}